API  Programmer Guide

- imprint(Revision V1.00).

*  All rights reserved, Specifications  subject to change without notice.

2. explain of backtrack value.

2.1  the function return values

0x00	Command OK. ( success)

0x01                      Command FAILURE, the error code is:

0x02  					 checksum error.

0x04  					 time out reply

0x05  					 check sequence error

0x07  					 Receive error

0x0A   					 the parameter value out of range

2.2  System Error/Status Codes (0x80-0xFF)

0x80                 SET OK. ( success)

0x81	SET FAILURE

0x82	Reader reply time out error

0x83	the card do not exist

0x84	the data is error

0x85	the authentication failure

0x87	operation error

0x8a                 In the block for the card initialization command error

0x8b                 in Anti-collision get the wrong serial number

0x8c                 Password Authentication do not pass

0x8f                 input cmd is not exisit

0x90                 show the card could not support this command

0x91 		 	     show the command format have a mistake

0x92                 show the command could not support OPTION form

0x93                 show the inputed block is inexistence.

0x94                 show the inputed block had been locked

0x95                 show Locked the block is not successful

0x96                 show the write card operation is not successful.

3、System Commands

3.1 	int  SetSerNum(

unsigned char    *newValue,

unsigned char     *buffer);

Description:

set  8 byte serial number which be supplied by manufactory

Input Parameter Description：

*newValue       8 byte serial number

*buffer          buffer send a pointer, it is used to return the received value.

Output Paramete:

If Command Failure

*buffer       System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE

3.2  int   GetSerNum(

unsigned char   *buffer);

Description:

read  one byte reader address and 8 byte serial number which be supplied by manufactory

Input Parameter Description：

*buffer          buffer send a pointer, it is used to return the received value.

Output Paramete:

*buffer         buffer[0]   reader address

buffer[1...8]   8 byte reader serial number

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE

3.3  int  WriteUserInfo(

int           num_blk,

int           num_length,

char          *user_info);

Description:

the reader consist of  4 blocks(each block less than 120 byte),the user data space in all 480 byte. the user could base the requirement to deposited the relevant userinfo into the reader.

Input Parameter Description：

num_blk         the block number

num_length       data length

*user_info        user information

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE

3.4  int   ReadUserInfo(

int           num_blk,

int           num_length,

char           *user_info);

Description:

Read the date from the reader, the reader consist of  4 blocks

(each block less than 120 byte)

Input Parameter Description：

int num_blk      the block number

int num_length    data length

*user_info        wait for reading user date

Output Paramete:

*user_info        If Command Failure,then user_info[0] is error code.

If Command success,then user_info[0..N] is read user information

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE

3.5 int   GetVersionNum(

char           *VersionNum);

Description:

read reader version number

Input Parameter Description：

*VersionNum    wait for reading version number

Output Paramete:

*VersionNum       If Command Failure,then VersionNum [0] is error code.

if Command success,then VersionNum [0..N] is reading version number

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE

3.6 int  ControlLED(

unsigned char		freq,

unsigned char 		duration,

unsigned char 		*buffer);

Description: the work state of set light,include,light Cyc and repeating times

Input Parameter Description：

freq            periodicity

duration         times

*buffer          wait for return value parameter

Output Paramete:

*buffer          If Command Failure,then buffer [0] 为 is error code.

If Command success ,then buffer [0] is 0x80

Return value:

0x00	Command OK. ( success)

0x01    Command FAILURE

3.7   int   API_ControlBuzzer(

unsigned char 			freq,

unsigned char 			duration,

unsigned char 			*buffer);

Description: the work state of setting buzzer,include buzzer work Cyc and repeating times

Input Parameter Description：

freq            periodicity

duration         times

*buffer          wait for return value parameter

Output Parameter:

*buffer          If Command Failure,,then buffer [0] 为 is error code.

If Command success,then buffer [0] is 0x80

Return value:

0x00	Command OK. ( success)

0x01    Command FAILURE

4  ISO14443 Type-A Commands

4.1  Type-A Commands

4.1.1 int  MF_Request(

unsigned char 	inf_mode,

unsigned char 	*buffer);

Description:： send ISO14443 A seeking card instruction

Input Parameter       Description：

Inf_mode           seeking card mode

0x01 –Idle mode（operate one card once a time）

0x00 –All mode（operate a lot of card once a time）

*buffer          wait for return value parameter

Output           Paramete:

*buffer          If Command FAILURE,then buffer [0] is error code.

if Command OK,then buffer [0..1]，return data bunch within 2 byte

Return value:

0x00	Command OK. ( success)

0x01    Command FAILURE

4.1.2 int MF_Anticoll(

unsigned  char    	*snr,

unsigned char    	&status);

Description:：test card quantity, single or much more, and then return the card number within 4 byte(if there are many cards, the only return one of card number

Input Parameter     Description：

&Status           send a finger，return to the number of the card

*snr              send a finger，return to 4 byte card number

Output Paramete:

If Command success

Status           the checked card number(0x00 means have checked a single card,0x01 means have checked more cards

*snr             4 byte card number（snr[0..3]）

If Command Failure

*snr             System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	          Command OK. ( success)

0x01              Command FAILURE

4.1.3  int   MF_Select(

unsigned char     *snr);

Description:

Select card, make the card as been select stat…

Input Parameter Description：

*snr            send a finger,input 4 byte card number,and return to 4 byte card numbe

Output Paramete:

Status           checked  card numbe

*snr            4 byte  card number4（snr[0..3]）

Return value:

0x00	       Command OK. ( success)

0x01           Command FAILURE

4 .1.4 int   MF_Halt( );

Description:

select card,make the card as been break off

Input Parameter Description：

None.

Return value:

0x00	   Command OK. ( success)

0x01       Command FAILURE

## 4.2    Mifare Appilication Commands

4.2.1 int  MF_Read(    unsigned char 		mode,

unsigned char        blk_add,

unsigned char 		num_blk,

unsigned char        *snr,

unsigned char 		*buffer);

Description:

read the appointed length date at the appointed station

Input Parameter Description：

mode，         read mode

( Request Idle + Key A    mode=00 ,  Request Idle + Key B     mode= 02,

Request All  + Key A    mode=01 ,  Request All  + Key B     mode=03）

（the up number is hex）

blk_add，      read block address

num_blk，      read block amount

*snr，          a finger, transfer eight byte secret key

*buffer         wait receive the variable of output finger

Output Paramete:

If Command success

*snr，     4 byte card number

*buffer，   the read date (the fact number is： num_blk*16)

If Command Failure

buffer[0]   System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01    Command FAILURE

4.2.2 int  MF_Write(

unsigned char   		mode,

unsigned char   		blk_add,

unsigned char   		num_blk,

unsigned char   		*snr,

unsigned char   		*buffer);

Description:

Read-in date At appoint station

Input Parameter Description：

mode，        needs write mode

( Request Idle + Key A    mode=00 ,  Request Idle + Key B     mode= 02,

Request All  + Key A    mode=01 ,  Request All  + Key B     mode=03）

blk_add，      needs fill in block address

num_blk，     needs fill in block number

*snr，         wait fill in date

*buffer，       afferent pointer sign to output the date

Output Paramete:

If Command success

snr[0..3]，      four byte card number

If Command Failure

buffer[0]        System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01    Command FAILURE

4.2.3 int   MF_InitVal(

unsigned char   mode,

unsigned char   SectNum,

unsigned char   *snr,

int             value);

Description:

Initialize  card

Input Parameter Description：

mode，        initialize mode

( Request Idle + Key A    mode=00 ,  Request Idle + Key B     mode= 02,

Request All  + Key A    mode=01 ,  Request All  + Key B     mode=03）

SectNum,       need initialize fan number

*snr,           six byte secret key (introduction as pointer)

value          4 byte  initialize  the date

Output Paramete:

If Command success

snr[0..3]      4 byte card number

If Command Failure

snr[0]        System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01    Command FAILURE

4.2.4 int  MF_Dec(

unsigned char   	 mode,

unsigned char   	 SectNum,

unsigned char   	 *snr,

int        		 *value);

Description:

Devalue work to the fan of the card

Input Parameter Description：

mode，         work mode

( Request Idle + Key A    mode=00 ,  Request Idle + Key B     mode= 02,

Request All  + Key A    mode=01 ,  Request All  + Key B     mode=03）

SectNum,       the fan number which need to write value 00-0F

*snr,           6 byte  secret key（introduction as pointer）

value          need decrease value,4 byte length

Output Paramete:

If Command success

snr[0..3]，      4 byte  card number

value[0..3]      date bunch after 4 byte work

If Command Failure

snr[0]         System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01    Command FAILURE

4.2.5 int 	MF_Inc (

unsigned char   mode,

unsigned char   SectNum,

unsigned char   *snr,

int        	  *value);

Description:

Add value work to appointed fan of the card.

Input Parameter Description：

mode，         work mode

( Request Idle + Key A    mode=00 ,  Request Idle + Key B     mode= 02,

Request All  + Key A    mode=01 ,  Request All  + Key B     mode=03）

SectNum,       need add value fan number 00-0F

*snr,           6 byte secret key( introduction as pointer)

value          need add value, 4 byte length

Output Paramete:

If Command success

snr[0..3]，      4 byte card number

value[0..3]      the date bunch after 4 byte work

If Command Failure

snr[0]          System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01    Command FAILURE

4.2.6 int  MF_Getsnr (

unsigned char       mode,

unsigned char       halt,

unsigned char       *snr

unsigned char 	  *value);

Description:

Return 1byte single card or numerous card sign，4 byte card number.

Input Parameter Description：

mode，        mode command  （mode command 26 or 52 ）

0x26 –Idle mode（one time only work to one card）

0x52 –All mode（one time can work to many card）

halt,     whether need halt card （halt select  00 or  01 ）

00   don’t need perform halt order

01   reader perform halt order

*snr,        returned 1byte single card or numerous card sign (if read card fail,return error code)

*value          return 4byte card number

Output Paramete:

If Command success

snr[0]，      1 byte single card or numerous

value[0..3]    return 4 byte card number

If Command Failure

snr [0]        System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01    Command FAILURE

4.2.7 int  MF_Restore(

unsigned char      mode,

int			     cardlength,

unsigned char 	 *carddata );

Description:

According the selected mode to send the date

Input Parameter Description：

mode，        mode command     0x00 —don’t need check CRC

0x01 —need check CRC

cardlength,      card date length

*carddata,       send time（card date）

incept time（return date）

Output Paramete:

If Command success

carddata[0..N],      incept the return date

If Command Failure

carddata[0]        System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01    Command FAILURE

# 5. ISO14443 Type-B Commands

5.1 int  RequestType_B(

unsigned char   *buffer)

Description: this order perform REQB order of ISO14443B, get PUPI code of the card

Input Parameter Description：

*buffer,         the date bunch after card reposition   （ATQB）

Output Parameter:

If Command success

*buffer,        the date bunch after card reposition   （ATQB）

buffer[0]        card reposition date length

buffer[0..N]      the date after work（ATQB）

If Command Failure

buffer[0]       System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01    Command FAILURE

5.2  int 	AntiType_B(

unsigned char       *buffer);

Description: this order perform Anticol1B of ISO14443B

Input Parameter Description：

*buffer,        The date after return card（ATQB）

Output Paramete:

If Command success

Buffer[0..N],        the date of card return（ATQB）

If Command Failure

buffer[0]       System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01    Command FAILURE

5.3 int  SelectType_B (

unsigned char     *SerialNum);

Description:

this order perform ATTRIB of ISO14443B, distribute a sign  for CID to the know card

Input Parameter Description：

*SerialNum,     the serial number of the card

Return value:

0x00	Command OK. ( success)

0x01    Command FAILURE

5.4 int   Request_AB(

unsigned char     * buffer);

Description:

This order performation integration  REQUEST AND ATTRIB order of ISO14443B,use one order to make card reposition.

Input Parameter Description：

* buffer,       return to 4 byte serial number of the worked card

Output Paramete:

If Command success

buffer[0..3],       return to 4 byte serial number of the worked card

If Command Failure

buffer[0]       System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01    Command FAILURE

5.5 int  API_ISO14443TypeBTransCOSCmd(

unsigned char   	 *cmd,

int              cmdSize,

unsigned char     *buffer);

Description:

ISO14443 remit order, any effect order and date can be transfer by this order

Input Parameter Description：

*cmd,         date which wait to be send

cmdSize,       date length

* buffer,       return date

Output Parameter:

If Command success

buffer[0..N]       the date which returned from the card

If Command Failure

buffer[0]      System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE

# 6  ISO15693 COMMANDS

6.1 	int   ISO15693_Inventory(

unsigned char          *Cardnumber,

unsigned char          *pBuffer);

Description:

This order is to get all the card’s serial number which in the reading card district（the getable card number is relate to the output rate of the module  antenna, commonly can read 2~6 card within anticollision）

Input Parameter Description：

*Cardnumber     the return card number(length is 1 byte)

*pBuffer        the return date（include LAG,  DSFID and 8*n byte card number）

Output Paramete:

IF：Command success (return 0x00)

*nrOfCard      return card number (one byte)

*pBuffer        return date（include FLAG,  DSFID and 8*n byte card number）

IF：Command Failure (return 0x01)

*nrOfCard		System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE, the error code is 2.2

6.2 			 int  ISO15693_Read	(

unsigned char     flags,

unsigned char     blk_add,

unsigned char     num_blk,

unsigned char     *uid,

unsigned char     *buffer);

Description:

It’s for reading 1 and numerous fan value. If need to read the safe digit of every block, make Option flag of FLAGS as 1 ,means FLAG=0x42,every fan will return 5 byte, include one byte show safe state and 4 byte block content, here the most can read 12 block, IF FLAG=02,will only return 4 byte block content,  here the most can read 63 block.

Input Parameter Description：

flags           0x02  without uid

0x22    with uid

0x42  without uid but need to read the safe digit

blk_add,       needed reading origin block number

num_blk,       block quantity

*uid           UID message

*buffer         return value

Output Paramete:

If : Command success (return 0x00)

*buffer          return date

buffer[0]  		return flag

buffer[1..N]  		Data

If : Command Failure (return 0x01)

buffer[0]             System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE, the error code is 2.2

6.3 int   ISO15693_Write(

unsigned char   flags,

unsigned char   blk_add,

unsigned char   num_blk,

unsigned char   *uid,

unsigned char   *data);

Description:    make writting work to every block(every time only can write one block)

Input Parameter Description：

flags          0x02  without uid

0x22    with uid

0x42  without uid but need to read the safe digit

blk_add,       needed writting origin block number

num_blk,       writing block quantity

*uid           UID message

*data          write data

Output Paramete:

If : Command success (return 0x00)

*buffer        return date

buffer[0]  	  return flag

buffer[1..N]  		Data

If : Command Failure (return 0x01)

buffer[0]       System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE, the error code is:2.2

6.4 int   ISO15693_Lock(

unsigned char   flags,

unsigned char   num_blk,

unsigned char   *uid

unsigned char   *buffer);

Description: for lock block content, caution : this process can not in reverse(can not unlock) the content can not revise when the block be locked.

Input Parameter Description：

flags          0x02  without uid

0x42  without uid but need to read the safe digit

0x22    with uid

num_blk,       locked block number

*uid           UID message

*buffer         return value

Output Paramete:

If : Command success (return 0x00)，

buffer[0]      return 0x80 ，means work ok.,

If : Command Failure (return 0x01)

buffer[0]      System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE, the error code is:2.2

6.5 int  ISO15693_StayQuiet(

unsigned char   flags,

unsigned char   *uid,

unsigned char   *buffer );

Description:

This order is to make the card to sleep sate, must use the address mode if the sent date is same to the card serial number, after this work finish, the card will be sleep, otherwise the state will not change.

Input Parameter Description：

flags         sign byte 1byte

*uid           UID message

*buffer         return value

Output Parameter:

If : Command success (return 0x00)

buffer[0]   return 0x80，means work ok.

If : Command Failure (return 0x01)

buffer[0]  System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE, the error code is:2.2

6.6 int  ISO15693_Select(

unsigned char   flags,

unsigned char   *uid,

unsigned char   *buffer ) ;

Description:

This order must use address mode, if the sent date is same to the card serial number, after the work ok, the card will be selected, otherwise the state will not change

Input Parameter Description：

flags          sign byte 1 byte

*uid           UID message

*buffer         return value

Output Paramete:

If : Command success (return 0x00)

buffer[0]   	return to 0x80，means work ok

If : Command Failure (return 0x01)

buffer[0]       System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE, the error code is 2.2

6.7 int  ISO15693_ResetToReady(

unsigned char       flags,

unsigned char 	  *uid,

unsigned char       *buffer );

Description:

After the work ok, the card will return to Ready state .

Input Parameter Description：

flags          sign byte 1 byte

0x02  without uid

0x42  without uid but need to read safe digit

0x22    with uid

*uid          UID message

*buffer        return value

Output Paramete:

If : Command success (return 0x00)

buffer[0]  			return 0x80，means work ok

If : Command Failure (return 0x01)

buffer[0]             System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE, the error code is:

6.8 int  ISO15693_WriteAFI(

unsigned char 	flags,

unsigned char  	afi,

unsigned char 	*uid,

unsigned char  	*buffer );

Description:

Write AFI to the card .

Input Parameter Description：

flags          sign byt 1 byte

0x02  without uid

0x42  without uid but need to read the safe digit

0x22  with uid

afi           wait write AFI

*uid          UID message

*buffer        return value

Output Paramete:

If : Command success (return 0x00)

buffer[0]             return 0x80，means work ok

If : Command Failure (return 0x01)

buffer[0]             System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE, the error code is:2.2

6.9 int  ISO15693_LockAFI(

unsigned char       flags,

unsigned char       *uid,

unsigned char       *buffer );

Description:

For lock AFI of the card, after lock AFI can not change

Input Parameter Description：

flags            sign byte 1 byte

0x02  without uid

0x42  without uid but need to read the safe digit

0x22  with uid

*uid          UID message

*buffer        return value

Output Paramete:

If : Command success (return 0x00)，

buffer[0]           return 0x80，means work ok

If : Command Failure (return 0x01)

buffer[0]   		System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE, the error code is:2.2

6.10 int  ISO15693_WriteDSFID(

unsigned char   flags,

unsigned char   DSFID,

unsigned char   *uid,

unsigned char   *buffer );

Description:

Write DSFID  to the card

Input Parameter Description：

flags          sign byte 1 byte

0x02   without   uid

0x42   without   uid but need to read the safe digit

0x22    with    uid

DSFID       the writted DSFID byte, the length is 1 byte

*uid          UID message

*buffer        return value

Output Paramete:

If : Command success (return 0x00)，

buffer[0]           return 0x80，means work ok

If : Command Failure (return 0x01)

buffer[0]   		System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE, the error code is:

6.11   int   ISO15693_LockDSFID(

unsigned char   flags,

unsigned char   *uid,

unsigned char   *buffer );

Description:

For lock DSFID of the card, after lock, DSFID can not change

Input Parameter Description：

flags          sign  byte (length is 1 byte )

0x02   without  uid

0x42  without uid

0x22    with uid

*uid          UID message

*buffer        return value

Output Parameter:

If : Command success (return 0x00)，

buffer[0]           return 0x80，means work ok

If : Command Failure (return 0x01)

buffer[0]   		System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE, the error code is:2.2

6.12   int  ISO15693_GetSysInfo(

unsigned char   flag,

unsigned char   *uid,

unsigned char   *Buffer);

Description:

For get the particular message of the card, the fact content please refer to ISO15693 agreement date.

Input Parameter Description：

flags          sign byte 1 byte

0x02   without uid

0x42   withou uid but need to read the safe digit

0x22   with  uid

*uid          UID message

*buffer        return value

Output Paramete:

If : Command success (return 0x00)，

Then Buffer [0]:	Flags

Buffer[1]:	INFO Flags

Buffer[2..9]:	UID

Buffer[10]:	DSFID

Buffer[11]:	AFI

Buffer[12..N]:     Other fields

If : Command Failure (return 0x01)

Buffer[0]   		System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE, the error code is:2.2

6.13  int  ISO15693_GetMulSecurity(

unsigned char 	   flag,

unsigned char 	   blkAddr,

unsigned char 	   blkNum,

const unsigned char   *uid,

unsigned char 	   *pBuffer);

Description:  for get the date of every safe state block of the card

Input Parameter Description：

flag           0x02  without uid

0x22    with uid

0x42  without uid but need to read the safe digit

blkAddr,       the read origin block number

blkNum,       the read block quantity

*uid           UID message

*pBuffer        return value

Output Paramete:

If : Command success (return 0x00)，

*pBuffer    return value

pBuffer [0]  return flags   pBuffer [1..N]  Block security status (the safe state of block)

If : Command Failure (return 0x01)

pBuffer[0]    		System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE, the error code is:2.2

6.14   int    ISO15693_TransCOSCmd(

unsigned char      *cmd,

int               cmdSize,

unsigned char      *buffer);

Description:  a currency order, the user can use this order to make kinds of handle to the card

Input Parameter Description：

*cmd,         the date need to be send.

cmdSize,       the date length

*buffer        return value

Output Paramete:

If : Command success (return 0x00)，

*buffer        return data

buffer [0..N]    the date return from the card

If : Command Failure (return 0x01)

buffer[0] 		System Error/Status Codes(You can consult the 2.2)

Return value:

0x00	Command OK. ( success)

0x01                      Command FAILURE, the error code is:2.2